| 1 Absolute ramp 1: Control mode |  |  |  |  | Displacement |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 Absolute ramp 1: End of ramp measurement |  |  |  |  | Force |  |  |
| 1 Absolute ramp 1: End of ramp type |  |  |  |  | Measurement value |  |  |
| 1 Absolute ramp 1: End of ramp value |  |  |  |  | 950.00 N |  |  |
| 1 Absolute ramp 1: Rate |  |  |  |  | 1.00 mm/min |  |  |
| 1 Absolute ramp 1: Step 1 name |  |  |  |  | Absolute ramp 1 |  |  |
| 1 Absolute ramp 1: Transition time |  |  |  |  | 0.00 s |  |  |
| 1 Absolute ramp 1-Step completion: Description |  |  |  |  | Step completion |  |  |
| 1 Absolute ramp 1-Step completion: Type |  |  |  |  | Step completion |  |  |
| 1 Absolute ramp 1-Step completion-Go to step: Description |  |  |  |  | Go to step |  |  |
| 1 Absolute ramp 1-Step completion-Go to step: Go to |  |  |  |  | Next step |  |  |
| 1 Absolute ramp 1-Step completion-Go to step: Loop |  |  |  |  | False |  |  |
| 1 Absolute ramp 1-Step completion-Go to step: Type |  |  |  |  | Go to step |  |  |
| 1 Data: Data capture scheme |  |  |  |  | Method scheme |  |  |
| Compressive strain (Displacement): Primary source |  |  |  |  | Displacement |  |  |
| Data: Criteria 1 |  |  |  |  | True |  |  |
| Data: Data capture scheme |  |  |  |  | Default settings |  |  |
| Data: Interval 1 |  |  |  |  | 20.00 ms |  |  |
| Data: Measurement 1 |  |  |  |  | Time |  |  |
| End of test: Criteria 1 |  |  |  |  | Measurement rate |  |  |
| End of test: End of test action |  |  |  |  | Stop |  |  |
| End of test: Measurement 1 |  |  |  |  | Force |  |  |
| End of test: Release pneumatic grips |  |  |  |  | False |  |  |
| End of test: Sensitivity 1 |  |  |  |  | 40.00 % |  |  |
| End of test: Set digital output lines |  |  |  |  | False |  |  |
| Force: Transducer configuration |  |  |  |  | Force |  |  |
|  |  |  | Page 1 of 2 |  |  |  |  |

|  |  | Page 2 of 2 |  |  |  |
| --- | --- | --- | --- | --- | --- |